# A Review of CNN-Based Approaches for Traffic Violation Detection and Automatic License Plate Recognition.

Introduction

The advent of AI and deep learning has revolutionized traffic management by introducing the possibility of automated, scalable, and highly accurate enforcement systems [PAPER- 5]. At the core of this transformation are Convolutional Neural Networks (CNNs), which excel at image and video analysis [PAPER- 4], [PAPER- 5]. CNN-based systems can monitor traffic feeds in real-time to automatically detect violations, identify offending vehicles through Automatic License Plate Recognition (ALPR).

This review covers 34 papers on CNN Based Approaches for Traffic Violation Detection and Automatic License Plate Recognition released in recent 5 years on trusted platforms like IEEE, ResearchGate, Arxiv, IJRIAS etc. The main Objective of this review Paper is to evaluate and identify the best performing CNN-based object detection systems available as of 2025, Based on criteria such as detection accuracy, speed, versatility, and real-world applicability.

Object Detection

The most crucial step in violation detection is identifying and classifying objects such as vehicles, pedestrians, and traffic signs within a video frame.

CNN-Based Object Detectors – In most of the recent traffic violation detection systems, it is seen that almost 95% of the modern systems requires real-time object detection.

- YOLO (You Only Look Once): YOLOv3, v5, v8, and v12 is frequently cited for its exceptional balance of speed and accuracy, making it ideal for real-time applications. It is used to detect a wide range of violations, from helmet non-compliance to red-light running. Most significantly cited in PAPER [1,4,9]

- SSD (Single Shot MultiBox Detector): Another one-stage detector praised for its efficiency, SSD is used for both general vehicle detection and as a core component in segmentation-free ALPR systems [PAPER- 6], [PAPER- 3].

- RFB-Net: An advanced one-stage detector inspired by the Receptive Field Block in human vision, it has been shown to deliver high accuracy and speed for license plate detection [PAPER- 20].

The YOLO family of models consistently demonstrates high performance. For instance in PAPER 5, a comparative study of Different versions of YOLO was carried out under different light settings, and it effectively yields good results :

The GA-YOLOv5 model also performs well, achieving a precision of 95.4% and a recall of 94.8%, with a faster speed of 45 FPS and good low-light support.

Accuracy comparison [PAPER 5]

The LoLTV model delivers outstanding performance with a precision of 98.2% and a recall of 97.5%, operating at 35 frames per second (FPS) and offering excellent low-light support. The GA-YOLOv5 model also performs well, achieving a precision of 95.4% and a recall of 94.8%, with a faster speed of 45 FPS and good low-light support. The CNN + LSTM model provides a precision of 91.1% and a recall of 89.2%, running at 22 FPS with moderate low-light capabilities. Faster-RCNN, while accurate with 92.5% precision and 91.0% recall, is slower at 12 FPS and offers poor low-light support. Lastly, Transformer models maintain a good balance with 93.8% precision, 92.3% recall, 18 FPS speed, and good performance in low-light conditions.)

Performance Metrics

- The YOLO family of models consistently demonstrates high performance. For instance, a YOLOv3-based helmet detection system achieved over 95.2% accuracy [PAPER- 1], while a YOLOv8-based model for Egyptian license plates reached 99.4% recognition accuracy [PAPER- 13]. Another YOLOv8 model specialized for low-light conditions (LoLTV) achieved 98.2% precision [PAPER- 8].

- A comparative study found that while YOLOv8 achieved a slightly higher mAP (0.995) for plate detection, YOLO-NAS was significantly faster, with an inference speed of 16.1ms versus 33.9ms for YOLOv8 [PAPER- 17].

- A comprehensive ALPR system using a custom YOLO-based network for Singaporean plates achieved 86% precision in plate detection and 67% accuracy in full character recognition in challenging "in the wild" conditions [PAPER- 20].

- A CNN-based system for Indian license plates reported an overall accuracy of 98.13%, showcasing high performance on multi-line, skewed, and multi-font plates [PAPER- 11].

Datasets High-quality, diverse datasets are critical for training robust models.

- General Datasets: The COCO (Common Objects in Context) dataset is widely used for pre-training general-purpose object detectors [PAPER- 6].

- Traffic-Specific Datasets: For specialized tasks, datasets like LoLTV (low-light violations in India) [PAPER- 8], AI City Challenge (urban traffic analysis) [PAPER- 5], and the EALPR dataset (Egyptian license plates) [PAPER- 13] are used.

- Custom Datasets: Many studies highlight the need for custom-collected and annotated datasets to address specific regional license plates (e.g., Romania, Singapore) or unique violation types [PAPER- 16], [PAPER- 20].

Challenges and Limitations

Despite promising results, the real-world deployment of CNN-based traffic monitoring systems faces several challenges.

- Environmental Factors: System accuracy is significantly impacted by variable lighting (especially low-light and night-time conditions), adverse weather, and motion blur [PAPER- 4], [PAPER- 6]. Glare and shadows can also severely degrade performance [PAPER- 8, PAGE -2].

- Data and Hardware: A major bottleneck is the scarcity of large-scale, diverse, and region-specific datasets [PAPER- 5], [PAPER- 14]. Furthermore, running complex deep learning models in real-time requires significant computational power, which can be costly and difficult to implement on low-power edge devices like a Raspberry Pi [PAPER- 5].

- ALPR Reliability: OCR performance often degrades on license plates that are dirty, damaged, occluded, or use non-standard fonts—a common issue in many regions [PAPER- 5]. Recognizing plates from multiple countries and layouts (e.g., single vs. double line) remains a significant challenge [PAPER- 14].

- Privacy and Ethics: The use of pervasive surveillance technology raises significant data privacy concerns [PAPER- 3], [PAPER- 5]. There are also risks of algorithmic bias, where a system may unfairly target certain types of vehicles or neighborhoods, necessitating robust legal and ethical frameworks to govern their use [PAPER- 3].

REFERENCES

[PAPER 1] A Deep Learning Approach for Helmet Detection and Fine Generation System, Sulbha Yadav et al., 2025

[PAPER 2] An Efficient and Layout-Independent Automatic License Plate Recognition System Based on the YOLO detector, Rayson Laroca et al.,  2021

[PAPER 3] Automated Traffic Violation Detection Using AI and Machine Learning, Angelina Grace, 2025

[PAPER 4] Automatic Traffic Rules Violation Detection and Number Plate Recognition System for Bangladesh, Raian Shahrear et al.,  2020

[PAPER 5] AI-Based Integrated Traffic Violation Detection and Smart Traffic Management System: A Comprehensive Review, Avadhut Dilip sutar, 2025

[PAPER 6] Deep Learning Based Segmentation Free License Plate Recognition Using Roadway Surveillance Camera Images, Alperen Elihos et al., (Year Not Available)

[PAPER 7] Deep Learning based Traffic Infringement Recognition System, Khallikkunaisa et al., 2023

[PAPER 8] LoLTV: A Low Light Two-Wheeler Violation Dataset With Anomaly Detection Technique, Samprit Bose et al., 2023

[PAPER 9] NEXT-GEN TRAFFIC SURVEILLANCE: AI-ASSISTED MOBILE TRAFFIC VIOLATION DETECTION SYSTEM, Dila Dede et al., 2023

[PAPER 10] Real Time Traffic Signal Violation using Artificial Intelligence and Machine Learning, Ajit Chaurasiya et al., 2022

[PAPER 11] TRAFFIC SIGNAL VIOLATION DETECTION USING ARTIFICIAL INTELLIGENCE AND DEEP LEARNING, Dr. S. Raj Anand et al., 2021

[PAPER 12] Two-Wheeler Traffic Violations Detection and Automated Penalty Issuance System, Batti Tulasidasu et al., 2025

[PAPER 13] UA-DETRAC: A New Benchmark and Protocol for Multi-Object Detection and Tracking, Longyin Wen et al., 2020

[PAPER 14] Automatic Detection of Traffic Violations Using Yolo Model and Challan Generation, Kishan Singh et al., 2024

[PAPER 15] An Intelligent System for Traffic Violation Detection: A Deep Learning Approach, M. Jayanthi et al., 2021

[PAPER 16] An Automated System to Detect Helmet on Motorcyclists and Pillion Rider Using YOLOv5 and Deep SORT, Arsalan Hasnain Khan et al., 2023

[PAPER 17] Real Time Traffic Signal Violation Detection System using Machine Learning, Anshul Sharma et al., 2021

[PAPER 18] Real-Time Automatic Number Plate Recognition based on YOLOv5, Ayush Roy et al., 2022

[PAPER 19] An Automatic Number Plate Recognition System to Monitor the Traffic Rule Violation, C. M. Rahman et al., 2021

[PAPER 20] Real-time Traffic Violation Detection System based on YOLO and Centroid Tracking, Yifan Wang et al., 2020

[PAPER 21] A Novel Approach for Traffic Violation Detection using Deep Learning, Ashish Kumar et al., 2021

[PAPER 22] Automatic Traffic Violation Detection: A Smart and an Intelligent Solution, Vishnu B et al., 2021

[PAPER 23] An Automated System for Monitoring of Traffic Rule Violations using Deep Learning, H.N. Harish et al., 2022

[PAPER 24] Smart Traffic Violation Detection and Alert System using Image Processing, Shubham Gupta et al., 2020

[PAPER 25] YOLO Based Advanced Smart Traffic Assistance Platform for Number Plate and Helmet Detection, K. Venkateswarlu et al., 2023

[PAPER 26] License Plate Detection Based on Improved YOLOv8n Network, Ruizhe Zhu et al., 2025

[PAPER 27] Fast Helmet and License Plate Detection Based on Lightweight YOLOv5, Chenyang Wei et al., 2023

[PAPER 28] End-to-End High Accuracy License Plate Recognition Based on Depthwise Separable Convolution Networks, Song-Ren Wang et al., 2022

[PAPER 29] Deep Learning Based Framework for Iranian License Plate Detection and Recognition, Mojtaba Shahidi Zandi & Roozbeh Rajabi, 2022

[PAPER 30] Cloud-Based License Plate Recognition: A Comparative Approach Using You Only Look Once Versions 5, 7, 8, and 9 Object Detection, Christine Bukola Asaju et al., 2025

[PAPER 31] Automated Indonesian Plate Recognition: YOLOv8 Detection and TensorFlow-CNN Character Classification, Windu Gata et al., 2025

[PAPER 32] A Systematic Review and Comparative Analysis Approach to Boom Gate Access Using Plate Number Recognition, Asaju Christine Bukola et al., 2024

[PAPER 33] A Robust Attentional Framework for License Plate Recognition in the Wild, Linjiang Zhang et al., 2020